# Software Outline — Senior Design

## Software files

- Current_Complete_Product.ino: the integrated product. Runs the white strobe, green chaser, and tone sweep while the trigger is held, and prints mic readings over serial.

- White-LED-Strobe.ino: prototype of the white LED strobe patterns.

- Green-LED-Strobe.ino: prototype of the rotating, blinking green LED chaser.

- Speaker_Setup.ino: prototype of random speaker volume patterns, with pitch tied to volume.

- Light_Log_Without_SD_W_Comments.ino: runs the light routine and logs every LED change over serial.

- Light_Log_With_SD.ino: runs the light routine and logs every LED change to an SD card.

- Sound_Log_Without_SD_Comments.ino: logs microphone readings over serial.

- Sound_Log_With_SD_Comments.ino: logs microphone readings to an SD card.

- Software.code-workspace: VS Code workspace settings; not part of the firmware.

## Current_Complete_Product.ino

### Functions

- setup(): sets pins 2–13 as outputs and A0 (trigger), A1 (mic) as inputs; starts serial at 115200 baud.

- loop(): while A0 is HIGH, runs one of ten random strobe patterns, then runGreen() and speaker(). When A0 goes LOW, turns pins 2–13 off, stops the tone, resets the green state, and prints a mic reading.

- flicker(thisPin, duration, brightness): strobes thisPin for duration / 70 on/off cycles of freqWhite ms each, running runGreen() and speaker() during the waits. Returns early if A0 goes LOW.

- nbDelay(ms): non-blocking pause; waits ms while running runGreen() and speaker(). Returns early if A0 goes LOW.

- runGreen(): every rotSpeed ms, moves the three-pin green window one pin along pins 3–12; every freqGreen ms, toggles the window on or off.

- speaker(): prints a mic reading (time, raw, dB); every sweep ms, plays speakerFreq on pin 13 and raises it 100 Hz, wrapping from 3000 back to 1000 Hz.

- logEvent(type, pin, stateStr): prints a CSV line (time, type, pin, state) while A0 is HIGH. Currently not called.

### Key variables

- lowestPin, highestPin (2, 13): range of output pins set up and cleared.

- freqWhite (40 ms): white strobe half-period, giving 12.5 Hz.

- freqGreen (35 ms): green blink half-period, about 14.3 Hz.

- rotSpeed (28 ms): time per one-pin step of the green window.

- state (0 or 1): whether the green window is lit.

- startPin (3–12): first pin of the three-pin green window.

- speakerFreq (1000–3000 Hz): current speaker tone.

- sweep (50 ms): time between tone steps.

- lastTime1, lastTime2, lastTime3 (ms): last update times for green rotation, green blink, and tone step.

- sensorValue (0–1023): raw microphone reading from A1.

- dB (30–134): mic reading rescaled as 30 + (sensorValue / 1023) × 104.

## White-LED-Strobe.ino

### Functions

- setup(): sets pins 2–13 as outputs and A0 (trigger), A1 (mic) as inputs.

- loop(): while A0 is HIGH, picks a random pattern with random(0, 9) and runs its three strobe bursts and pauses using delay().

- flicker(thisPin, duration, brightness): strobes thisPin for duration / 70 on/off cycles of freq ms each.

### Key variables

- lowestPin, highestPin (2, 13): range of output pins set up.

- freq (40 ms): strobe half-period, computed as 1000 ÷ (2 × desired Hz).

- thisPin, duration, brightness: global copies of the flicker() parameter names; not used.

## Green-LED-Strobe.ino

### Functions

- setup(): sets pins 2–13 as outputs and A0 (trigger), A1 (mic) as inputs.

- loop(): while A0 is HIGH, moves the three-pin green window one pin every rotSpeed ms and toggles it on or off every freq ms.

### Key variables

- lowestPin, highestPin (2, 13): range of output pins set up.

- freq (35 ms): green blink half-period.

- rotSpeed (28 ms): time per one-pin step of the window.

- state (0 or 1): whether the window is lit.

- startPin (3–12): first pin of the three-pin window.

- lastTime1, lastTime2 (ms): last update times for rotation and blink.

- thisPin, duration, brightness: carried over from the white prototype; not used.

## Speaker_Setup.ino

### Functions

- setup(): sets pins 13 (speaker) and 12 (volume) as outputs and A0 as input, writes the starting volume, and seeds random() from unconnected pin A3.

- loop(): while A0 is HIGH, starts a random volume pattern if none is active, then runs volume() and speaker(). When A0 goes LOW, stops the tone, sets volume to 0, and clears the pattern.

- speaker(): every sweep ms, maps volumeLevel (100–255) to a pitch of 3000–10000 Hz and plays it on pin 13.

- volume(): looks up the current step's level and duration in one of ten three-step patterns, limits the level to 100–255, and writes it as PWM on pin 12. After each step's duration it advances; after the third step it picks a new random pattern.

### Key variables

- speakerPin, volumePin, triggerPin (13, 12, A0): speaker output, volume PWM output, trigger input.

- minimumVolume, maximumVolume (100, 255): limits for the volume level.

- minimumFrequency, maximumFrequency (3000, 10000 Hz): pitch range mapped from the volume range.

- sweep (50 ms): how often the pitch is updated.

- volumeLevel (100–255): current PWM volume level.

- volumePattern (−1 or 0–9): active pattern; −1 means none.

- volumeStep (0–2): current step within the pattern.

- volumePatternStart (ms): time the current step began.

- lastTime3 (ms): time of the last pitch update.

## Light_Log_Without_SD_W_Comments.ino

### Functions

- setup(): sets pins 2–13 as outputs and A0, A1 as inputs; starts serial at 115200 baud.

- loop(): while A0 is HIGH, runs one of ten random strobe patterns, then runGreen().

- flicker(thisPin, duration, brightness): strobes thisPin as in the main product and logs each on and off.

- nbDelay(ms): waits ms while running runGreen().

- runGreen(): runs the green chaser as in the main product and logs each LED change.

- logEvent(type, pin, stateStr): prints a CSV line: time, LED type, pin, ON or OFF.

### Key variables

- lowestPin, highestPin (2, 13): range of output pins set up.

- freqWhite, freqGreen, rotSpeed (40, 35, 28 ms): white strobe half-period, green blink half-period, and green step time.

- state (0 or 1), startPin (3–12): whether the green window is lit, and its first pin.

- lastTime1, lastTime2 (ms): last update times for green rotation and blink.

## Light_Log_With_SD.ino

### Functions

- setup(): sets pins 2–13 as outputs and A0, A1 as inputs; starts serial at 9600 baud for error messages. Starts the SD card, opens strobe.txt, and writes the header row; halts if either fails.

- loop(): while A0 is HIGH, runs one of ten random strobe patterns, then runGreen(), and flushes the log file if at least 500 ms have passed.

- flicker(thisPin, duration, brightness): strobes thisPin as in the main product and writes each on and off to the file.

- nbDelay(ms): waits ms while running runGreen().

- runGreen(): runs the green chaser as in the main product and writes each LED change to the file.

### Key variables

- chipSelect (10): SD card chip select pin.

- logFile: the open strobe.txt log file.

- lastFlush (ms): time the file was last flushed to the card.

- lowestPin, highestPin (2, 13): range of output pins set up.

- freqWhite, freqGreen, rotSpeed (40, 35, 28 ms): white strobe half-period, green blink half-period, and green step time.

- state (0 or 1), startPin (3–12): whether the green window is lit, and its first pin.

- lastTime1, lastTime2 (ms): last update times for green rotation and blink.

## Sound_Log_Without_SD_Comments.ino

### Functions

- setup(): starts serial at 115200 baud, sets A0 and A2 as inputs and pin 13 as output, and prints the header row Time(ms), Raw, dB.

- loop(): while A0 is HIGH, reads the mic, prints time, raw value, and dB, plays a 200 Hz tone on pin 13, and waits 200 ms. Stops the tone otherwise.

### Key variables

- micPin (A2): microphone input.

- triggerPin (A0): trigger input; logging runs only while it is HIGH.

- chipSelect (10): SD chip select pin; declared but not used in this serial version.

- sensorValue (0–1023): raw microphone reading.

- dB (30–134): mic reading rescaled as 30 + (sensorValue / 1023) × 104.

## Sound_Log_With_SD_Comments.ino

### Functions

- setup(): starts serial at 9600 baud, sets A0 and A2 as inputs, and starts the SD card, halting if it fails. Deletes the old log.txt and writes the header row Time(ms), Raw, dB.

- loop(): while A0 is HIGH, reads the mic, opens log.txt, appends time, raw value, and dB, closes the file, and waits 200 ms.

### Key variables

- micPin (A2): microphone input.

- triggerPin (A0): trigger input; logging runs only while it is HIGH.

- chipSelect (10): SD card chip select pin.

- myFile: the log.txt file, opened and closed for each sample.

- sensorValue (0–1023): raw microphone reading.

- dB (30–134): mic reading rescaled as 30 + (sensorValue / 1023) × 104.